Below is a complete v1 calculation engine spec (formulas + tables + step-by-step algorithm) and the measurement instructions needed to generate a high-quality fit for:

- Road (drop bar)

- Gravel (drop bar)

- MTB (flat bar)

- City / commute (flat bar)

It is designed to be implemented directly in your app (Convex/Next stack or any backend), and it explicitly blends these common fit methods:

- LeMond saddle height baseline (inseam-based)

- Holmes-style knee angle targets (as a validation step after setup)

- KOPS (as a field check for saddle fore–aft, not a “law”)

- Stack & Reach coordinate system (for frame size + stem/spacer solving)

# 0) Definitions, units, and reference points (non-negotiable)

Units:

- All lengths in mm

- Angles in degrees

## Bike coordinate system (for calculations)

- Origin = Bottom Bracket center (BB)

- +X = forward

- +Y = upward

## App output reference points

These must be consistent to avoid confusion:

### Saddle height (SH)

BB center → saddle top, measured along the seat tube line (tape measure along seat tube).

### Saddle setback (SS)

Horizontal distance from BB vertical line → saddle nose.

- Positive = saddle nose behind BB

- Negative = saddle nose in front of BB

### Bar drop (BD)

Vertical difference from saddle top → handlebar reference point.

- Drop bar bikes (road/gravel): reference point = top of bar at center near stem clamp

- Flat bar bikes (MTB/city): reference point = bar center at stem clamp
 (Your app can add a future “grip point” model using bar rise/sweep, but v1 should stay consistent.)

BD sign convention:

- Positive = bar below saddle

- Negative = bar above saddle

### Saddle-to-bar reach (SR)

Horizontal distance from saddle nose → bar center at stem clamp
 (Yes, even on drop bars; v1 uses one cockpit reach metric.)

# 1) Measurement instructions (body) — required for accurate fitting

## 1.1 Required body measurements

### A) Height (H)

- Barefoot, back to wall, neutral head, book on head, mark wall.

- Measure floor → mark.

Input: height_mm

### B) Cycling inseam (I) (critical)

- Barefoot, back to wall.

- Press a book firmly into the crotch (simulate saddle pressure).

- Measure floor → top edge of book.

Input: inseam_mm

## 1.2 Strongly recommended (improves reach + setback accuracy)

### C) Torso length (T)

- Sit upright on a hard chair.

- Place book firmly into crotch.

- Measure from sternal notch (top of breastbone) → top of book.

Input: torso_mm

### D) Arm length (A)

- Arm straight, relaxed.

- Measure shoulder joint center → wrist crease.

Input: arm_mm

### E) Shoulder width (SW)

- Measure bony outer shoulder points (acromion) left→right.

Input: shoulder_width_mm

### F) Foot length (FL)

- Heel to longest toe (standing).

Input: foot_length_mm

### G) Femur length (FEM) (optional but best for setback refinement)

- Sit with knee at 90°.

- Measure hip joint center (outer hip prominence) → knee center.

Input: femur_mm

# 2) Flexibility + core scoring protocol (0–10) with exact mappings

Your engine needs numerical flexibility and core inputs. Use these tests to generate standardized app scores.

## 2.1 Flexibility score (FS: 0–10)

Use Sit-and-Reach (hamstring + posterior chain proxy)

Test:

- Sit with legs straight, feet against wall/box

- Reach forward with both hands

- Measure reach relative to toes

  - If fingertips stop before toes: negative value

  - If past toes: positive value

Mapping table (exact):

| Sit-and-reach result | Flex Score (FS) |
| --- | --- |
| ≤ -10 cm | 0 |
| -9 to -7 cm | 1 |
| -6 to -4 cm | 2 |
| -3 to -1 cm | 3 |
| 0 to +2 cm | 4 |
| +3 to +5 cm | 5 |
| +6 to +9 cm | 6 |
| +10 to +14 cm | 7 |
| +15 to +19 cm | 8 |
| +20 to +24 cm | 9 |
| ≥ +25 cm | 10 |

## 2.2 Core stability score (CS: 0–10)

Use front plank hold (quality form)

Test rules:

- Straight line head→heels

- No sagging, no piking

- Stop when form breaks

Mapping table (exact):

| Plank time | Core Score (CS) |
| --- | --- |
| < 20 s | 0 |
| 20–29 s | 1 |
| 30–39 s | 2 |
| 40–49 s | 3 |
| 50–59 s | 4 |
| 60–74 s | 5 |
| 75–89 s | 6 |
| 90–109 s | 7 |
| 110–139 s | 8 |
| 140–179 s | 9 |
| ≥ 180 s | 10 |

# 3) Ambition (goal) input and normalization

User selects:

- Comfort

- Balanced

- Performance

- Aero (road/gravel only; for MTB/city treat as Performance)

Represent as:

type Ambition = "comfort" | "balanced" | "performance" | "aero";

Normalize indices used by engine:

flexIndex = FS - 5;   // -5..+5

coreIndex = CS - 5;   // -5..+5

# 4) Bike category constants (numerical tables)

## 4.1 Category setup constants (v1)

| Category | Saddle height multiplier Msh | Base setback formula | Base drop ratio by ambition | Base reach offset OR |
| --- | --- | --- | --- | --- |
| Road | 0.883 | SS0 = 0.070*I - 5 | see table 4.2 | 0 |
| Gravel | 0.880 | SS0 = 0.070*I - 5 | see table 4.2 | -10 |
| MTB | 0.875 | SS0 = 0.070*I - 5 | see table 4.2 | -60 |
| City | 0.870 | SS0 = 0.070*I - 5 | see table 4.2 | -120 |

Notes:

- SS0 produces a realistic starting saddle-nose setback range for most adult riders.

- Category offsets (below) fine-tune this.

## 4.2 Bar drop ratio table (exact)

Compute base drop as:

BD0 = SH * Rbd(category, ambition)

Where Rbd is:

| Category | Comfort | Balanced | Performance | Aero |
| --- | --- | --- | --- | --- |
| Road | 0.07 | 0.10 | 0.13 | 0.16 |
| Gravel | 0.05 | 0.07 | 0.09 | 0.11 |
| MTB | 0.01 | 0.03 | 0.05 | 0.05 |
| City | -0.06 | -0.04 | -0.02 | -0.02 |

Interpreting City ratios: negative drop = bars above saddle.

## 4.3 Saddle setback category + ambition offsets (exact)

Final setback starts from SS0 then adds offsets:

### Category offsets ΔSS_cat

| Road | Gravel | MTB | City |
| --- | --- | --- | --- |
| +0 | +5 | -5 | +10 |

### Ambition offsets ΔSS_amb

| Comfort | Balanced | Performance | Aero |
| --- | --- | --- | --- |
| +6 | 0 | -6 | -18 |

(Aero = more forward position; reduces setback.)

## 4.4 Saddle tilt targets (exact)

Expressed as degrees relative to level (nose-down negative).

| Category | Base tilt |
| --- | --- |
| Road | -1.0° |
| Gravel | -1.0° |
| MTB | 0.0° |
| City | 0.0° |

Drop-based modifier:

- If BD > 90mm: add -0.5°

- If BD > 120mm: add another -0.5°

- Clamp tilt range to [-3.0°, +1.0°]

## 4.5 Cleat fore–aft target offsets (exact)

Define CleatOffset = pedal axle behind ball of foot (mm). Positive means cleat is moved rearward.

| Category | Comfort | Balanced | Performance | Aero |
| --- | --- | --- | --- | --- |
| Road | 8 | 6 | 3 | 5 |
| Gravel | 10 | 8 | 6 | 7 |
| MTB | 14 | 12 | 10 | 10 |
| City | 16 | 14 | 12 | 12 |

## 4.6 Handlebar width targets (exact)

Compute baseline road bar width:

HB_road = roundToNearest20(shoulder_width_mm)

Then category adds:

| Category | Add (comfort) | Add (balanced) | Add (performance/aero) |
| --- | --- | --- | --- |
| Road | +0 | +0 | +0 |
| Gravel | +40 | +30 | +20 |
| MTB | use MTB formula below | use MTB formula below | use MTB formula below |
| City | +80 | +60 | +40 |

MTB formula (flat bar):
 HB_mtb = clamp(720, 820, roundToNearest10(1.80 * shoulder_width_mm))

City formula (flat bar):
 HB_city = clamp(600, 740, roundToNearest10(1.60 * shoulder_width_mm))

# 5) Core calculation engine (exact step-by-step)

## 5.1 Input schema (minimum viable)

type BikeCategory = "road" | "gravel" | "mtb" | "city";

type Ambition = "comfort" | "balanced" | "performance" | "aero";

type FitInputs = {

category: BikeCategory;

ambition: Ambition;

height_mm: number;

inseam_mm: number;

flexibility_score: number; // 0..10

core_score: number;        // 0..10

// recommended (optional)

torso_mm?: number;

arm_mm?: number;

shoulder_width_mm?: number;

foot_length_mm?: number;

femur_mm?: number;

// optional bike geometry for frame/stem solving

frame_stack_mm?: number;

frame_reach_mm?: number;

stem_length_mm?: number;   // current

stem_angle_deg?: number;   // current

spacer_mm?: number;        // current

// optional current measurements for comparison

current_saddle_height_mm?: number;

current_setback_mm?: number;

current_drop_mm?: number;

current_reach_mm?: number;

};

## 5.2 Validation rules (reject or warn)

Hard rejects:

- height_mm not in [1300, 2100]

- inseam_mm not in [550, 1050]

- inseam_mm >= height_mm (impossible)

Warnings (ask to remeasure):

- Inseam ratio out of normal band: inseam/height < 0.40 or > 0.52

- If torso_mm provided and torso_mm < 450 or > 750

- If arm_mm provided and arm_mm < 450 or > 750

## 5.3 Step 1 — Crank length recommendation (exact)

Use inseam bands:

| Inseam (mm) | Crank length (mm) |
| --- | --- |
| < 740 | 165 |
| 740–819 | 170 |
| 820–879 | 172.5 |
| 880–939 | 175 |
| ≥ 940 | 177.5 |

Category tweak:

- MTB: if inseam ≥ 820, prefer -2.5 mm option if available (e.g., 172.5 instead of 175) for clearance

- City: keep default band choice

## 5.4 Step 2 — Saddle height (SH) (exact)

### Base:

SH0 = inseam_mm * Msh(category)

### Flex/core modifiers:

ΔSH_flex = clamp(-8, +4, 1.5 * (FS - 5));

ΔSH_core = clamp(-4, +2, 0.8 * (CS - 5));

### Ambition modifier:

| Comfort | Balanced | Performance | Aero |
| --- | --- | --- | --- |
| -4 | 0 | +3 | +6 |

(If category is MTB/city and ambition=aero, treat as performance.)

### Final:

SH = SH0 + ΔSH_flex + ΔSH_core + ΔSH_amb

Clamp safety band (engine-level guardrails):

- Minimum: inseam * 0.865

- Maximum: inseam * 0.890

If outside, clamp and emit warning.

### Validation step (Holmes-style, practical)

After setting SH, instruct user:

- Record a short side video while pedaling steady.

- Target knee flexion at bottom dead center (BDC):

  - Comfort/City/MTB: 35–37°

  - Road/Gravel Balanced: 33–35°

  - Performance/Aero: 30–34°

- Adjust saddle height in 2–3 mm increments.

(Your app can keep the computed SH as “starting point” and the knee angle as the “validation”.)

## 5.5 Step 3 — Saddle setback (SS) (exact + KOPS field check)

### Base:

SS0 = 0.070 * inseam_mm - 5

### Add category + ambition offsets:

SS1 = SS0 + ΔSS_cat + ΔSS_amb

### Femur refinement (optional but exact)

If femur_mm present:

femurRatio = femur_mm / inseam_mm;

ΔSS_femur = clamp(-15, +15, (femurRatio - 0.55) * 300);

Interpretation:

- Longer femur vs inseam → more setback

- Shorter femur vs inseam → less setback

### Flex/core refinements (small but meaningful)

ΔSS_flex = clamp(-5, +3, 0.8 * (FS - 5));   // low flex => move slightly forward

ΔSS_core = clamp(-5,  0, 0.8 * (CS - 5));   // low core => move forward, never backward

### Final:

SS = SS1 + ΔSS_femur + ΔSS_flex + ΔSS_core
 (If no femur: omit ΔSS_femur.)

Clamp SS ranges by category (guardrails):

- Road: 20 to 90 mm

- Gravel: 25 to 95 mm

- MTB: 0 to 75 mm

- City: 30 to 110 mm

### KOPS check (numerical targets per category)

This is your app’s “field calibration” instruction:

How to measure KOPS:

- Put crank at 3 o’clock (forward and horizontal)

- Place rider seated normally with hands on bars

- Drop a plumb line from front of kneecap (patella)

Target kneecap position relative to pedal axle:

| Category | Comfort | Balanced | Performance | Aero |
| --- | --- | --- | --- | --- |
| Road | -10 to -20 mm | -5 to -15 mm | -5 to +5 mm | +5 to +20 mm |
| Gravel | -10 to -20 mm | -5 to -15 mm | -5 to +5 mm | 0 to +15 mm |
| MTB | -5 to -15 mm | -5 to +5 mm | 0 to +10 mm | 0 to +10 mm |
| City | -15 to -30 mm | -10 to -25 mm | -10 to -20 mm | -10 to -20 mm |

Negative = kneecap behind axle. Positive = in front.

Adjustment rule for users:
 If kneecap is too far forward → increase SS (move saddle back).
 If too far back → decrease SS (move saddle forward).
 Use 5 mm steps.

## 5.6 Step 4 — Bar drop (BD) (exact)

### Base:

BD0 = SH * Rbd(category, ambition) (from table 4.2)

### Flex/core modifiers:

ΔBD_flex = clamp(-25, +15, 5.0 * (FS - 5));

ΔBD_core = clamp(-20, +10, 4.0 * (CS - 5));

### Experience modifier (optional input, recommended for UX)

If you add experience_level:

- beginner: -10 mm

- intermediate: 0

- advanced: +5 mm

### Final:

BD = BD0 + ΔBD_flex + ΔBD_core + ΔBD_exp

Clamp by category:

- Road: 40..150

- Gravel: 20..120

- MTB: -10..70

- City: -140..40

## 5.7 Step 5 — Saddle-to-bar reach (SR) (exact)

### Base reach computation

If torso and arm exist:

SR0 = 0.47 * torso_mm + 0.33 * arm_mm + 25

If missing torso/arm (fallback):
 SR0 = 0.33 * height_mm

### Apply category offset:

SR1 = SR0 + OR(category)
 (From table 4.1: road 0, gravel -10, mtb -60, city -120)

### Ambition offsets:

| Comfort | Balanced | Performance | Aero |
| --- | --- | --- | --- |
| -20 | 0 | +10 | +25 |

### Flex/core offsets:

ΔSR_flex = clamp(-20, +10, 3.0 * (FS - 5));

ΔSR_core = clamp(-20, +10, 3.0 * (CS - 5));

### Final:

SR = SR1 + ΔSR_amb + ΔSR_flex + ΔSR_core

Clamp ranges:

- Road: 470..630

- Gravel: 450..610

- MTB: 380..540

- City: 300..480

## 5.8 Step 6 — Saddle tilt (ST) (exact)

ST = baseTilt(category) + dropModifiers(BD) then clamp [-3.0, +1.0]

(From table 4.4.)

## 5.9 Step 7 — Cleat position (exact)

CleatOffset_mm = table(category, ambition) from table 4.5.

Measurement instruction to implement in-app:

- Remove shoe insole

- Find 1st metatarsal head (ball of foot) by pressing from inside

- Mark it on outside of shoe

- Install cleat so pedal axle is CleatOffset_mm behind that mark

- Start with neutral rotation (match natural toe-out)

## 5.10 Step 8 — Handlebar width (exact)

As in table 4.6.

# 6) From fit targets → frame size + stem/spacer solution (stack & reach logic)

This is how the app becomes commercially valuable: it turns body + goals into frame size and cockpit parts.

## 6.1 Compute target bar coordinates relative to BB

Approximate saddle vertical height:

- If seat tube angle unknown, use:
 saddleY = 0.96 * SH

- Saddle nose X coordinate:
 saddleX = -SS

Then:

- barX_target = saddleX + SR

- barY_target = saddleY - BD

These are your target cockpit coordinates.

## 6.2 Solve stem + spacers if frame stack/reach are known

Given:

- frame_stack_mm (FSK)

- frame_reach_mm (FRH)

- stem_length_mm (L)

- stem_angle_deg (α)

- spacer_mm (SP)

- topcap_mm (TC) assume default 10 mm if unknown

Compute bar clamp coordinates:

barX_actual = FRH + L * cos(α);

barY_actual = FSK + TC + SP + L * sin(α);

Error:

errX = barX_actual - barX_target;

errY = barY_actual - barY_target;

Optimization strategy (implementation-ready):

- Candidate stems: L ∈ {60,70,80,90,100,110,120,130}

- Candidate angles:

  - Road/Gravel: {-17, -10, -6, 0, +6}

  - MTB/City: {-6, 0, +6, +10, +17}

- Spacers: SP ∈ {0,5,10,15,20,25,30,35,40}

Pick the combo minimizing:
 score = sqrt(errX^2 + errY^2)
 with constraints:

- Road/Gravel: prefer SP ≤ 35 (stability)

- MTB/City: SP ≤ 40

Output:

- best stem_length, stem_angle, spacer_mm

- residual errors (tell user: “within 5 mm” etc.)

## 6.3 Frame size recommendation if frame is unknown (geometry DB integration)

Choose category “default cockpit parts” (v1 assumptions):

| Category | Default stem L | Default stem angle | Default spacers |
| --- | --- | --- | --- |
| Road | 100 | -6° | 15 |
| Gravel | 90 | -6° | 20 |
| MTB | 50 | 0° | 10 |
| City | 80 | +10° | 25 |

Then compute target frame stack/reach:

frame_reach_target = barX_target - L*cos(α);

frame_stack_target = barY_target - TC - SP - L*sin(α);

Recommend frames with:

- reach within ±10 mm

- stack within ±15 mm

(That’s exactly how you query a geometry database.)

# 7) Measurement instructions (bike) per category

These are app steps the rider follows after receiving target numbers.

## 7.1 Road + Gravel (drop bar) — how to measure

- Saddle height: BB center → saddle top (along seat tube)

- Setback: plumb line from saddle nose; measure horizontal to BB center line

- Bar drop: vertical from saddle top → top of bar near stem

- Saddle-to-bar reach: horizontal saddle nose → bar center at stem

Adjustment order:

- Cleats → 2) Saddle height → 3) Saddle setback → 4) Bar drop (spacers) → 5) Reach (stem)

## 7.2 MTB — how to measure

Pre-step: set suspension sag to manufacturer recommendation (so cockpit height is realistic).

Measure:

- Saddle height with dropper fully extended

- Bar drop uses bar center at stem clamp

- Reach uses saddle nose → stem clamp center

MTB-specific note:

- Keep saddle slightly lower than road to reduce “over-extension” on rough terrain (already baked into multiplier + drop tables).

## 7.3 City — how to measure

Same as MTB (flat bar reference at stem clamp).

City-specific note:

- If frequent stops, your UX can optionally add a “stop frequency” slider that reduces SH by 5–15 mm, but warn about knee load if too low.

# 8) Numerical injury-risk rules (engine warnings)

Trigger warnings if:

### Saddle height risk

- SH > inseam*0.890 → likely too high (posterior knee/hamstring risk)

- SH < inseam*0.865 → likely too low (anterior knee risk)

### Drop risk

- If FS <= 3 and BD > 90 → lumbar/neck strain risk

- If CS <= 3 and BD > 80 → shoulder/neck + core fatigue risk

### Reach risk

- If CS <= 3 and SR > (categoryMax-10) → excessive reach risk

- If user reports hand numbness → recommend +10 mm stack (less drop) first, not shorter saddle height

# 9) What your app should output (per category, exact)

For every rider, output:

- SH saddle height (mm)

- SS saddle setback (mm behind BB)

- ST saddle tilt (deg)

- BD bar drop (mm; can be negative)

- SR saddle-to-bar reach (mm)

- crank_length_mm

- handlebar_width_mm

- cleat_offset_mm (axle behind ball)

Plus, if geometry DB is available:

- frame_stack_target_mm ±15

- frame_reach_target_mm ±10

- Suggested stem length/angle + spacer stack