# 1) Homepage — /

## SEO metadata

Title tag: BikeFit.ai | Bike Fit Calculator & Bike Setup Based on Your Measurements
 Meta description: Get precise bike setup recommendations—saddle height, setback, reach & drop—based on your body measurements, flexibility and riding style. Road, MTB and TT supported.

## Hero

### H1: Precision bike fit based on your body measurements

Intro paragraph:
 BikeFit.ai helps you set up your bike with clear, millimeter-level recommendations—so you can ride more comfortably, reduce the risk of fit-related pain, and feel more stable and efficient on the bike.

Supporting paragraph:
 Enter a few measurements, choose your riding style, and get your recommended saddle height, saddle setback, handlebar reach, handlebar drop, crank length and more—explained in a way you can actually apply at home.

Primary CTA: Start your fit
 Secondary CTA: How it works

Trust microcopy (under CTA):
 No account required to try. Takes about 3 minutes. Results in millimeters.

## “What you’ll get” section

### H2: Your setup recommendations (explained, not just numbers)

Paragraph:
 Most bike fit tools give you a single number and send you on your way. BikeFit.ai gives you the numbers and the “why” behind them—plus the safest order to apply changes.

Bullets (short, scannable):

- Saddle height (mm)

- Saddle setback (mm)

- Saddle-to-bar reach (mm)

- Saddle-to-bar drop (mm)

- Recommended crank length (mm)

- Suggested handlebar width (road + MTB guidance)

- Optional: compare to your current setup and see the difference

CTA: Open the bike fit calculator

## Style selector section

### H2: Built for how you actually ride

Paragraph:
 A mountain bike setup shouldn’t feel like a time trial position—and a TT setup shouldn’t be guessed from a generic road formula. Choose your riding style and BikeFit.ai adapts the recommendations accordingly.

Cards (copy per card):

- MTB Trail — More upright and stable for control, braking and technical terrain.

- Road Endurance — Comfort-first position for long rides and reduced strain.

- Road Race — More aggressive posture for performance while staying sustainable.

- Time Trial / Tri — Aero-focused position with careful drop/reach limits.

CTA: Start with your riding style

## How it works

### H2: How BikeFit.ai works

Step 1 — Measure
 You’ll measure height and inseam (and optionally torso, arms and femur). We show you exactly how with a simple guide.

Step 2 — Choose your riding style
 Road, MTB or TT changes what “good fit” means. We apply style-specific geometry targets.

Step 3 — Get your recommended setup (mm)
 We calculate your fit outputs and show how to translate them onto your bike.

Step 4 — Adjust safely
 We guide the order of changes and suggest small steps to help you adapt.

CTA: See the measurement guide

## Trust / methodology (E‑E‑A‑T builder)

### H2: Based on bike fitting principles—made practical

Paragraph:
 Bike fit is a mix of biomechanics and real-world constraints. BikeFit.ai uses proven fitting concepts (inseam-based saddle height, femur-informed setback starting points, and stack/reach cockpit logic) and then applies sensible limits based on your flexibility, stability and style.

Paragraph:
 You’ll always see clear definitions (what each number means), how to measure it on your bike, and what to adjust first.

CTA: Read the calculation approach

## For bike shops teaser

### H2: Bike shop tools (optional)

Paragraph:
 If you fit multiple riders, BikeFit.ai can become your fast first step—before you fine-tune with your preferred in-person fitting workflow.

CTA: BikeFit.ai for bike shops

## Homepage FAQ (short)

### H2: FAQ

Q: Is this a replacement for a professional bike fit?
 A: It’s a strong starting point and works very well for most riders. If you have persistent pain, unusual anatomy, or complex injury history, an in-person professional fit is recommended.

Q: What do I need to measure?
 A: At minimum: height, inseam, riding style, and your flexibility/core scores. Optional measurements improve reach/setback precision.

Q: Can I use it for MTB and TT?
 A: Yes. MTB and TT positions have different targets, and BikeFit.ai accounts for that.

Q: What if I don’t know my current setup?
 A: No problem. You can still get recommendations and learn how to measure your current bike later.

## Final CTA

### H2: Ready to set up your bike the right way?

Paragraph:
 Get your personalized bike fit recommendations in minutes.

CTA: Start your fit

## Homepage images (briefs for design)

- Hero image brief: rider silhouette + bike + overlay lines showing saddle height, setback, reach, drop

- Alt text: “Bike fit overlay showing saddle height, setback, reach, and drop measurements.”

- Caption: “Your position, explained in millimeters.”

- Style cards image brief: 4 small side-view silhouettes (MTB upright → TT aero)

- Alt text: “Cycling position silhouettes for MTB, endurance, race, and time trial.”

- Caption: “Different riding styles need different fits.”

# 2) Bike Fit Calculator — /bike-fit-calculator

## SEO metadata

Title tag: Bike Fit Calculator | BikeFit.ai
 Meta description: Calculate your bike setup from your body measurements: saddle height, setback, reach, drop, crank length and more. Includes measurement help and adjustment steps.

## Page header

### H1: Bike fit calculator

Intro paragraph:
 Use this bike fit calculator to generate setup recommendations based on your body measurements and riding style. You’ll get clear values in millimeters and guidance on how to apply them safely.

Trust microcopy:
 Best results when measurements are accurate. If you’re unsure, open the Measurement Guide in a new tab.

## Calculator module copy (above the form)

### H2: What you’ll need

Paragraph:
 At minimum you’ll enter your height and inseam, then choose a riding style. Optional measurements (torso, arms and femur) make reach and setback more personalized.

Bullets:

- Takes ~3 minutes

- Outputs in mm and degrees where relevant

- Includes an adjustment order (what to change first)

## Form section copy (labels + help text)

### H2: Step 1 — Your measurements

Field: Height (cm)
 Help text: Measure barefoot against a wall. Use centimeters.

Field: Inseam (cm)
 Help text: Use the cycling inseam method with a hardcover book pressed upward (see Measurement Guide).

Field: Torso length (optional, cm)
 Help text: Hip bone (iliac crest) to shoulder bone (acromion) while sitting upright.

Field: Arm length (optional, cm)
 Help text: Shoulder bone to wrist crease with arm relaxed and slightly forward.

Field: Femur length (optional, cm)
 Help text: Side hip bone (greater trochanter) to center of kneecap while seated.

Field: Shoulder width (optional, cm)
 Help text: Outside shoulder bone to outside shoulder bone (acromion-to-acromion).

### H2: Step 2 — Riding style

Field: Riding style
 Help text: Choose the style that matches your typical rides most of the time.
 Options (microcopy):

- MTB Trail — stable + upright

- Road Endurance — comfort-first

- Road Race — performance posture

- Time Trial / Tri — aero-focused

### H2: Step 3 — Mobility & stability

Field: Flexibility (1–5)
 Help text: Based on a simple sit-and-reach test. Higher scores can usually tolerate more drop.

Field: Core stability (1–5)
 Help text: Based on a plank test. Higher scores generally tolerate a longer reach more comfortably.

### H2: Optional — Compare to your current setup

Paragraph:
 If you know your current bike measurements, add them to see a clear difference between “current” and “recommended.”

Fields (optional):

- Current saddle height (mm)

- Current saddle setback (mm)

- Current saddle-to-bar reach (mm)

- Current saddle-to-bar drop (mm)

- Current crank length (mm)

## Button labels

Primary: Calculate my fit
 Secondary: Reset

Under button microcopy:
 Make changes in small steps. If pain appears, stop and reassess.

## Results section copy (after calculation)

### H2: Your recommended bike setup

Paragraph:
 These are starting-point recommendations designed to get you close quickly. Small refinements are normal.

Results labels (with short definitions):

- Saddle height (mm): Bottom bracket to top of saddle (along seat tube axis).

- Saddle setback (mm): Saddle nose behind the bottom bracket.

- Saddle-to-bar reach (mm): Horizontal distance from saddle nose to bar center.

- Saddle-to-bar drop (mm): Vertical difference from saddle top to bar top.

### H3: Recommended parts

- Crank length: Nearest standard size based on inseam.

- Handlebar width (guidance): Based on shoulder width and riding style.

## “How to apply adjustments” section

### H2: Apply changes safely (recommended order)

Paragraph:
 Adjustments interact with each other. For best results, use this order and keep changes small.

Step 1 — Set saddle height first
 Make changes in small increments (for example 2–5 mm). Test ride and re-check stability and knee comfort.

Step 2 — Set saddle setback
 Move the saddle fore/aft in small steps. Re-check how your knees track during pedaling and whether you feel too stretched or too cramped.

Step 3 — Adjust reach and drop
 Only after saddle position is stable, adjust cockpit (spacers/stem/lever position) to match your target reach and drop.

Step 4 — Fine-tune contact points
 Saddle tilt, hood/lever angle, and cleat position are finishing touches.

## Accuracy & limitations (trust section)

### H2: Accuracy, limitations, and when to get help

Paragraph:
 BikeFit.ai provides a strong baseline fit based on measurements and common fitting principles. However, it can’t fully account for unique anatomy, injuries, or advanced aerodynamic tradeoffs without an in-person assessment.

Paragraph:
 If you experience sharp pain, numbness that persists, or symptoms that worsen after adjustments, stop and consult a qualified bike fitter or medical professional.

## Calculator page images (briefs)

- Form illustration: simple checklist graphic with icons (tape measure, book, wall, stopwatch)

- Alt text: “Checklist of tools needed to measure for a bike fit at home.”

- Caption: “A tape measure + a book gets you most of the way.”

- Results illustration: diagram showing BB reference point with arrows for height/setback/reach/drop

- Alt text: “Bike fit diagram showing saddle height, setback, reach, and drop relative to the bottom bracket.”

- Caption: “Every number refers to a clear physical measurement.”

# 3) Measurement Guide — /measurement-guide

## SEO metadata

Title tag: Bike Fit Measurement Guide | How to Measure for BikeFit.ai
 Meta description: Step-by-step instructions to measure height, inseam, torso, arms, femur, shoulder width, flexibility and core stability for accurate bike fit recommendations.

## Page header

### H1: How to measure yourself for an accurate bike fit

Intro paragraph:
 Accurate inputs create accurate outputs. This guide shows how to measure each field BikeFit.ai uses—clearly, safely, and with common mistakes to avoid.

What you’ll need (short list):

- Tape measure

- Hardcover book (for inseam)

- Wall + flat floor

- A friend (recommended, not required)

- Phone camera (optional, for checking posture)

## Section: Height (cm)

### H2: Measure your height

What it is:
 Your standing height from floor to the top of your head.

How to measure:

- Stand barefoot with your back against a wall.

- Heels, hips and upper back lightly touching the wall.

- Look straight ahead (neutral head).

- Place a book flat on your head, touching the wall.

- Mark the wall and measure from the floor to the mark.

Common mistakes:
 Measuring in shoes, tilting the head up, measuring on carpet without a hard reference.

Image brief: person against wall with book on head and a measurement line

- Alt text: “Measuring height barefoot against a wall using a book as a straight edge.”

- Caption: “Keep your head neutral and measure from a hard floor.”

## Section: Inseam (cm)

### H2: Measure your cycling inseam (most important input)

What it is:
 Distance from the floor to your pubic bone—measured with pressure that simulates sitting on a saddle.

How to measure:

- Stand barefoot with your back against a wall.

- Place a hardcover book between your legs.

- Pull the book upward firmly (it should feel like “saddle pressure”).

- Keep the book level (parallel to the floor).

- Measure from the floor to the top edge of the book.

Common mistakes:
 Not applying enough upward pressure, using a soft object, measuring alone without keeping the book level.

Image brief: inseam method with book between legs + arrow from floor to top edge

- Alt text: “Cycling inseam measurement using a hardcover book pressed upward against the pubic bone.”

- Caption: “Firm upward pressure improves accuracy.”

## Section: Torso length (cm, optional)

### H2: Measure your torso length

What it is:
 A practical torso measurement used to personalize cockpit reach.

How to measure:

- Sit upright on a firm chair, feet flat.

- Find the top of your hip bone (iliac crest).

- Find the bony tip on top of your shoulder (acromion).

- Measure from iliac crest to acromion.

Common mistakes:
 Slouching, measuring to the collar line instead of the shoulder bone.

Image brief: seated upright with markers on hip bone and shoulder bone

- Alt text: “Torso length measurement from hip bone (iliac crest) to shoulder bone (acromion) while seated upright.”

- Caption: “Sit tall—don’t slouch.”

## Section: Arm length (cm, optional)

### H2: Measure your arm length

What it is:
 Shoulder bone to wrist crease distance—helps refine reach.

How to measure:

- Stand relaxed.

- Raise your arm slightly forward (not fully out to the side).

- Measure from the shoulder bone (acromion) to your wrist crease.

Common mistakes:
 Measuring from the neck instead of the shoulder bone, locking the elbow rigidly.

Image brief: arm slightly forward with measurement line shoulder→wrist

- Alt text: “Arm length measurement from shoulder bone to wrist crease with arm slightly forward.”

- Caption: “Measure from the shoulder bone, not the neck.”

## Section: Femur length (cm, optional)

### H2: Measure your femur length

What it is:
 Upper leg length; improves saddle setback personalization.

How to measure:

- Sit upright with knees bent at about 90 degrees.

- Find the bony point on the side of your hip (greater trochanter).

- Find the center of your kneecap.

- Measure from greater trochanter to kneecap center.

Common mistakes:
 Measuring from the waist, measuring to the top of the kneecap instead of center.

Image brief: seated with hip landmark and kneecap center highlighted

- Alt text: “Femur length measurement from greater trochanter (hip) to the center of the kneecap while seated.”

- Caption: “Use the side hip bone as your start point.”

## Section: Shoulder width (cm, optional)

### H2: Measure your shoulder width

What it is:
 Distance between the bony tips of your shoulders (acromion-to-acromion).

How to measure:

- Stand relaxed.

- Find the bony tips on the outside of each shoulder.

- Measure straight across from acromion to acromion.

Common mistakes:
 Measuring around the arms, measuring across the chest to the wrong points.

Image brief: back view with acromion points and a straight line between them

- Alt text: “Shoulder width measurement from acromion to acromion across the back.”

- Caption: “Measure bone-to-bone, not around the shoulders.”

## Section: Flexibility score (1–5)

### H2: Quick flexibility test (1–5)

What it is:
 A simple proxy for hamstring and lower-back mobility—used to scale handlebar drop recommendations.

How to do the test:

- Sit on the floor with legs straight.

- Keep knees straight and reach forward.

- Score yourself:

- 1: cannot reach knees

- 2: reaches mid‑shin

- 3: reaches ankles

- 4: touches toes

- 5: hands past toes comfortably

Common mistakes:
 Bouncing into the stretch, rounding aggressively to “cheat” the reach.

Image brief: seated reach with labeled distance targets

- Alt text: “Sit-and-reach flexibility test positions mapped to a 1–5 score.”

- Caption: “Comfortable range matters more than forcing distance.”

## Section: Core stability score (1–5)

### H2: Quick core stability test (1–5)

What it is:
 A proxy for how well you can support your pelvis and spine—used to scale sustainable reach and posture.

How to do the test:

- Hold a plank with neutral spine.

- Stop when form breaks (hips sag, shoulders collapse, breath control fails).

- Score:

- 1: under 20 seconds

- 2: 30 seconds

- 3: 60 seconds

- 4: 90 seconds

- 5: 120+ seconds with excellent form

Common mistakes:
 Holding longer with poor form, craning the neck upward.

Image brief: plank with “neutral spine” callout

- Alt text: “Plank test showing neutral spine alignment for core stability scoring.”

- Caption: “Stop when form breaks—quality over time.”

## Accuracy checklist (end of page)

### H2: Accuracy checklist (before you enter your numbers)

Bullets:

- Use centimeters for body measurements; use millimeters for bike measurements

- Measure on a hard, flat floor

- Repeat inseam twice and use the average

- If two measurements differ by more than 1 cm, measure again

- If you’re unsure, ask someone to help for 60 seconds—it improves accuracy a lot

CTA: Go to the bike fit calculator

# 4) Pricing — /pricing

## SEO metadata

Title tag: Pricing | BikeFit.ai
 Meta description: Choose the BikeFit.ai plan that matches your needs—free basic fit, Pro recommendations and advanced comparisons, and shop tools for multi-rider workflows.

## Page header

### H1: Pricing that fits your riding

Intro paragraph:
 Start with the essentials for free. Upgrade when you want deeper analysis, comparisons, and step-by-step adjustment guidance.

Note: If you already know your final pricing, replace the placeholders below. If not, this copy works with or without exact numbers.

## Plan cards (copy)

### Plan 1: Free

Tagline: Get a solid baseline fit in minutes
 Includes:

- Core fit outputs (saddle height + starting recommendations)

- Measurement definitions and how-to guidance

- Basic safety notes

CTA: Start free

### Plan 2: Pro

Tagline: The most complete fit experience
 Includes:

- Full setup recommendations (height, setback, reach, drop)

- Style comparison (MTB vs endurance vs race vs TT)

- Progressive adjustment plan (what to change first)

- Compare recommended vs current setup

- Export/share results (PDF or link) (if you support this feature—otherwise remove)

CTA: Upgrade to Pro

Microcopy:
 Best for riders training regularly or dialing in comfort/performance.

### Plan 3: Shop

Tagline: Built for bike shops and fitters
 Includes:

- Multi-rider dashboard

- Client history and fit exports

- Faster intake workflows

- Partner support

- Optional: white-label and API access (only include if offered)

CTA: Talk to sales

## Feature comparison (copy-first, no table required)

### H2: What’s included

Paragraph:
 Every plan focuses on clarity and safety: clear measurement definitions, results in millimeters, and guidance on applying changes without guessing.

Suggested feature bullets (grouped):
 Fit outputs

- Saddle height

- Saddle setback

- Reach and drop targets

- Crank length suggestion

- Handlebar width guidance

Guidance

- Measurement instructions

- Adjustment order

- Safety notes and basic warnings

Pro tools

- Current vs recommended comparison

- Style-based fit targets

- Export/share

Shop tools

- Multi-client workflow

- Reporting

- Integrations (if applicable)

## Pricing FAQ (high-conversion)

### H2: Pricing FAQ

Q: Can I try it for free?
 A: Yes. The Free plan helps you get a baseline fit. Upgrade when you want deeper cockpit guidance and comparisons.

Q: Do I need Pro for accurate results?
 A: The biggest accuracy driver is correct measurements. Pro adds more personalization inputs and more detailed recommendations.

Q: Can I cancel anytime?
 A: Yes. You can cancel at any time (billing terms depend on monthly/annual options you choose).

Q: Is this safe if I have pain?
 A: Fit changes can reduce discomfort, but persistent or sharp pain should be evaluated by a qualified professional. Always make changes in small steps.

## Final CTA

### H2: Start free—upgrade when you’re ready

Paragraph:
 Get your baseline fit now, then refine as your riding goals evolve.

CTA: Start free